SECTION 02100  CLEARING AND GRUBBING

1.
Be sure to include provisions for measurement and payment in some other item or on the bid form.

2.
You may delete this paragraph if you add Section 01025 "Measurement and Payment".

PART 1 - GENERAL

A.
Description
This section includes removal and disposal of all timber, brush, stumps, roots, grass, weeds, sawdust, rubbish, struc​tures, and existing pavement.

Calls for grubbing of areas under structures, pavement, fills or roadways.

B.
Measurement and Payment
Work under this section will not be paid for as a sepa​rate item unless a specific unit price basis is included in the Bid Form.

PART 2 - PRODUCTS

A.
Disposal of Materials
1.
Haul away and properly dispose of all excess materials.

PART 3 - EXECUTION

Calls for clearing of entire project area unless noted otherwise.

A.
Clearing
In all areas of the project, unless otherwise noted, remove and dispose of trees, snags, stumps, shrubs, brush, limbs, and other vegetative growth.  Remove all evidence of their presence from the surface including sticks and branches greater than 1 inch in diameter or thickness.  Remove and dispose of trash piles, rubbish and stockpiled organic mate​rial.  Protect trees, shrubs, and vegetative growth which are not designated for removal.

B.
Grubbing
In all areas where excavation will be done, in all areas under structures, roadways or pavements, in all areas which fill will be placed, remove all stumps, roots, brush and other debris greater than 1 inch in diameter to a depth of not less than two feet below the subgrade.

END OF SECTION

ORB9919000   July 24, 2002
CLEARING AND GRUBBING
02100-

FW-L52-100-04   July 24, 2002
CLEARING AND GRUBBING
02100-


